| Distillate Stream (Ethanol-Rich Product) |  |
| --- | --- |
| Composition | 95.6 wt% ethanol, 4.4 wt% water |
| Mol fraction of ethanol (xD) | 0.8948 |
| Temperature | 78°C |
| Density (ρD) | 806 kg/m³ |
| Average molecular weight (MD) | 43.12 g/mol |
| Molar flow rate (D) | 77.9 kmol/h |
| Mass flow rate (mD) | 3.358 kg/h |
| Volumetric flow rate | 100,000 L/day |
| Annual production | 3.30 × 107 L/year |

| Bottom Stream (Water-Rich Waste) |  |
| --- | --- |
| Composition | 0.1 wt% ethanol, 99.9 wt% water |
| Mol fraction of ethanol (xB) | 0.000391 |
| Temperature | 100°C |
| Density (ρB) | 958 kg/m³ |
| Average molecular weight (MB) | 18.02 g/mol |
| Molar flow rate (B) | 2,045 kmol/h |
| Mass flow rate (mB) | 36,72 kg/h |

| Feed Stream |  |
| --- | --- |
| Composition | 8.06 wt% ethanol, 91.94 wt% water |
| Mol fraction of ethanol (xF) | 0.0332 |
| Feed condition | Saturated liquid (q = 1) |
| Average molecular weight (MF) | 18.94 g/mol |
| Molar flow rate (F) | 2,123 kmol/h |
| Mass flow rate (mF) | 40,210 kg/h |
| Volumetric Flow Rate (VF) | 38.3 m3/h |

| Hammer Mill |  |
| --- | --- |
| Equipment Type | Dry-milling hammer mill |
| Raw Material Feed Rate | 12,170 kg/h (approx. 292 tons/day) |
| Feed Moisture Content | 14% |
| Bond’s Work Index (Wi) | 8.5 kWh/ton |
| Feed Particle Size (F80) | 5,000 μm |
| Product Particle Size (P80) | 400 μm |
| Calculated Shaft Power | 37.1 kW |
| Design Motor Specification | 40.0 kW |

| Hydrolysis Reactor |  |
| --- | --- |
| Process Operation | Continuous enzymatic hydrolysis (Liquefaction & Saccharification) |
| Brand Name | Cycle Columbus/OEM |
| Operating Temperature | 70 °C |
| Retention (Residence) Time | 6 hours |
| Safety Factor | 20% allowance included |
| Total Reactor Volume | 276 m³ |
| Mechanical Agitation Power | 138 kW |

| Fermenter System |  |
| --- | --- |
| Process Operation | Continuous anaerobic fermentation |
| Brand Name | BLBIO/Bailun |
| Jacket Tank Material | 316L stainless steel |
| Microorganism Culture | Saccharomyces cerevisiae |
| Retention (Residence) Time | 48 hours |
| Bioreactor Configuration | 4 parallel units operating simultaneously |
| Unit Bioreactor Volume | 552 m³ per vessel |
| Total Fermenter Volume | 2,205 m³ |
| Fermentation Efficiency | 90% |
| Starch-to-Glucose Conversion Efficiency | 95% |
| Exothermic Cooling Load | 3525 kW |
| Distillation Column |  |  |
| Column Type | Continuous Sieve Tray Column |  |
| Operating Pressure | 1 atm |  |
| Top Temperature | 78°C |  |
| Bottom Temperature | 100°C |  |
| Column Diameter (D) | 1.80 m |  |
| Column Height (H) | 13.0 m |  |
| Bottom clearance (sump) | 2 m |  |
| Top clearance (disengagement) | 1.5 m |  |
| Number of Theoretical Trays | 15 |  |
| Tray Efficiency (Murphree, EO) | 70% |  |
| Number of Actual Trays | 20 |  |
| Tray Spacing | 0.5 m |  |
| Downcomer Area (Adc) | 0.27 m2 |  |
| Maximum vapor velocity (umax) | 2.084 m/s |  |
| Operating vapor velocity (uop) | 1.667 m/s |  |
| Reflux ratio (R) | 4.42 |  |
| Minimum reflux ratio (Rmin) | 3.40 |  |
| Optimal Feed Tray Location | Stage 7 (from the top) |  |